CANPORT

PC to Controller Area Network Adapter

Application Documentation

Version 2.0

9/15/97

Table Of Contents

Introduction

Typical Applications

Design Intentions

CANPORT Overview

Hardware Overview

Software Overview

Installation

Software

CANPORT Library Functions

List of Functions

Function: qcSetPortAddress

Function: qcReset

Function: qcReadRegister

Function: qcWriteRegister

Function: qcSearchPort

Function: qcInitializeBaud

Function: qcSetAcceptance

Function: qcSetupMessage

Function: qcCheckMessage

Function: qcReadMessage

Function: qcResetMessage

Function: qcSendMessage

Function: qcOnline

Function: qcOffline

Function: qcWriteMessageData

Function: qcReadMessageData

Function: qcDataReceivedLen

CANPORT Sample Visual Basic Program

CANPORT Delphi Application

Excel CANPORT Application

Macros

Setup CANPORT

Messages

Registers

DOS

Libraries

Test Application

# Introduction

The CANPORT from Embedded Micro Software is a very economical way to connect a Personal Computer (PC) to a Controller Area Network (CAN). The CANPORT is designed to be easy to use and easy to design applications to interface to a CAN network. The CANPORT connects to the parallel printer port of the PC.

## Typical Applications

- Send and receive CAN messages

- Develop powerful CAN applications using PC software

- Compact design is ideal for portable applications

- Simulate network traffic to test module applications

## Design Intentions

The CANPORT is intended to be a low-cost alternative to the PCMCIA, PC Bus Cards, and serial interface cards used to connect a PC to the CAN network. In order to keep the cost low while maintaining a high degree of functionality, the CANPORT relies on the powerful software available for the PC.

The software included is general in nature and is intended to aid in getting your application up and running quickly.

## CANPORT Overview

The following block diagram shows the hardware and software model.

## Hardware Overview

The CANPORT uses an Intel 82527 CAN controller to connect to the network. The 82527 provides;

- Support for CAN Specification 2.0

- Standard 11 bit message identifier data and remote frames

- Extended 29 bit message identifier data and remote frames

- Global Identifier Masks (Acceptance filters)

- 14 Message Objects for transmit or receive

- 1 Receive Message Object with programmable mask

- Programmable Bit Rate

The physical layer is a Phillips 82C250.

## Software Overview

Low-level control of the CANPORT is provided by either using the CANPORT.DLL dynamic link library for Windows 95 or linking your DOS application to one of the libraries. The CANPORT.DLL or CANDOSx.LIB are used to control the interface through the parallel port to the 82527. Functions to initialize the CANPORT, initialize the network, send and receive messages are provided. The user’s application interfaces the CAN network using the CANPORT.DLL.

## Installation

The CANPORT interfaces to the parallel printer port of the PC. It is designed to work with both the standard uni-directional and newer bi-directional printer ports.

The following external connections are required;

- Connect the CAN network to CAN+ and CAN- on the green terminal block.

- Connect a DC voltage (6 to 24 volts) to V+ and GND on the green terminal block.

- Verify polarity is correct (CANPORT is protected for reverse polarity and overvoltage).

- Connect the CANPORT to the printer port on the PC.

- Turn on power.

- Test the installation by running the TEST.EXE program in a DOS window. The printer ports are searched and the connection verified.

- If CANPORT is not found, verify all connections. Also, use the debug command explained under qcSetPortAddress to identify the parallel port address.

## Software

The following software is included with the CANPORT;

- Windows 95 Dynamic Link Library (DLL)

- Sample source code using Visual Basic 5.0

- Sample source code using Borland Delphi 3.0

- Sample Application using Excel

- DOS Libraries

- Sample DOS Application

Software updates are provided free of charge from our web site at www.emicros.com. Send your email address to us at info@emicros.com and we will advise you when updates are available. We also encourage you to send us suggestions for enhancements at the same address.

## Directory of Files

- A:\CANPORT\CANPORT.DLL		Copy this file to \windows\system

- A:\CANPORT\EXCEL\CANPORT.XLS	Excel application

- A:\CANPORT\CANVB5			Visual Basic 5.0 sample application directory

- A:\CANPORT\CANDEL			Borland Delphi 3.0 sample application directory

- A:\CANPORT\CANDOS			DOS Libraries and sample application

# CANPORT Library Functions

The CANPORT dynamic link library (DLL) is called CANPORT.dll and is included on the installation disk. Install this file in the \windows\system directory. The following functions (or procedures) are used to access the CANPORT board, configure the network, send messages, and monitor messages. Each are explained in detail in the following sections.

## List of Functions

qcSetPortAddress		Sets the parallel port base address

qcReset			Resets the CANPORT board

qcReadRegister		Read one register on the 82527

qcWriteRegister		Write one register on the 82527

qcSearchPort		Search parallel port address for CANPORT board

qcInitializeBaud		Initialize the network baud rate

qcSetAcceptance		Sets the message acceptance filters

qcSetupMessage		Configures message objects

qcCheckMessage		Returns the status of a message

qcReadMessage		Read the message object data bytes

qcResetMessage		Reset the receive message flag

qcSendMessage		Sends a message

qcOnline			Enables CANPORT to send and receive messages

qcOffline			Removes CANPORT from sending and receiving messages

qcWriteMessageData	Writes one byte to the message object

qcReadMessageData	Reads one byte of a message object

qcDataReceivedLen		Reads the number of bytes received

## Function:	qcSetPortAddress

The base address of the printer port connected to the CANPORT must be established. The qcSetPortAddress function sets the base address of the parallel port. Use this function when the printer port address is known or if the function qcSearchPort returns an error.

Calling Conventions

C		unsigned int qcSetPortAddress(unsigned int port)

Pascal		function qcSetPortAddress( port: Integer ): integer; stdcall;

Function Return Codes:		qcSetPortAddress = 0	Initialization complete

Example:

qcSetPortAddress( 0x378 );

Common printer port assignments are 0x0378, 0x03bc, and 0x0278. To identify the printer port assignments for a particular machine, use the DOS debug program to display memory locations 0040:0008. For example:

C:\> debug

-d 0040:0008 L8

0040:0008	78  03  78  02  00  00  00  00

In this example LPT1 is at 0x0378 and LPT2 is at 0x0278 and LPT3 and LPT4 are not assigned.

## Function:	qcReset

The qcReset function resets the Intel 82527.

Calling Conventions

C		unsigned int qcReset(void)

Pascal		function qcReset: integer; stdcall;

Function Return Codes:		qcReset = 0	Initialization complete

qcReset = -1	Port address not initialized

Example:

if( qcReset() == -1 )

printf( “Port address not initialized. \n” )

else

printf( “CANPORT reset complete. \n” )

## Function:	qcReadRegister

The qcReadRegister function reads any one of the 82527 registers.

Calling Conventions

C		unsigned int qcReadRegister( unsigned char reg)

Pascal		function qcReadRegister( reg: Byte ): integer; stdcall;

Function Return Codes:		qcReadRegister = x	Valid register value range 0 to 0xfe

qcReadRegister = -1	Port address not initialized

NOTE: Register 0xFF is the serial reset address and should not be accessed.

Example:

void ReadRegisters()

{

int x,y;

for( x=0; x<0xfe; x++ )

{

y = qcReadRegister( x );

if( y < 0 )

{

printf( “Error in qcReadRegister()\n” );

break;

}

else

printf( “Reading Register %d %d\n”, x, y );

}

}

## Function:	qcWriteRegister

The qcWriteRegister function writes any one of the 82527 registers.

Calling Conventions

C		unsigned int qcWriteRegister( unsigned char reg, unsigned char dta )

Pascal		function qcWriteRegister( reg: Byte; dta: Byte ): integer; stdcall;

Function Return Codes:		qcWriteRegister = 0	Write successful

qcWriteRegister = -1	Port address not initialized

NOTE: Register 0xFF is the serial reset address and should not be accessed.

Example:	printf( “%d\n”, qcWriteRegister( 0x17, 0x05 ) );	// write data byte

## Function:	qcSearchPort

The qcSearchPort function searches the typical parallel port addresses for the CANPORT board. Search order is 0x378, 0x3bc, the 0x278. If this function cannot find the CANPORT then refer to the qcSetPortAddress section for details on establishing printer port address.

Calling Conventions

C		unsigned int qcSearchPort(void)

Pascal		function qcSearchPort: integer; stdcall;

Function Return Codes:		qcSearchPort = 0x378

qcSearchPort = 0x3BC

qcSearchPort = 0x278

qcSearchPort = -1		Not found

Example:

x = qcSearchPort();

if( x == -1 )

printf( “CANPORT not found!\n” );

else

printf( “CANPORT connected to address %x\n”, x );

## Function:	qcInitializeBaud

The qcInitializeBaud function initializes the baud rate.

Calling Conventions

C	unsigned int qcInitializeBaud( 	unsigned int BaudRatePrescaler,

unsigned int TimeSegment1,

unsigned int TimeSegment2,

unsigned int SynchronizationJumpWidth,

unsigned int SamplingMode)

Pascal	function qcInitializeBaud(	BaudRatePrescaler: integer,

TimeSegment1: integer,

TimeSegment2: integer,

SynchronizationJumpWidth: integer,

SamplingMode: integer): integer; stdcall;

BaudRatePrescaler		Valid range from 0 to 63

TimeSegment1		Valid range 2 to 15

TimeSegment2		Valid range 1 to 7

SynchronizationJumpWidth	Valid range 0 to 3

SamplingMode		0 = 1 sample,  1=3 samples

Function Return Codes:	-1 = Port not initialized

-4 = Invalid BaudRatePrescaler

-5 = Invalid TimeSegment1

-6 = Invalid TimeSegment2

-7 = Invalid SynchronizationJumpWidth

-8 = Invalid SamplingMode

Baud Rate = 8000000 / (2 * (BaudRatePrescaler + 1) * (3 + TimeSegment1 + TimeSegment2))

Refer to the 82527 Data Sheet for Baud Rate Settings.

## Function:	qcSetAcceptance

The qcSetAcceptance function initializes the standard and extended acceptance filters.

Calling Conventions

C		unsigned int qcSetAcceptance(unsigned int stdGlobalMask,

long extGlobalMask )

Pascal		function qcSetAcceptance( stdGlobalMaskBaudRatePrescaler: integer,

extGlobalMask: longint ): integer; stdcall;

Function Return Codes:		qcSetAcceptance = 0	Successful

qcSetAcceptance = -1	Port address not initialized

qcSetAcceptance = -12	Invalid standard mask ( > 0x7FF)

qcSetAcceptance = -13	Invalid extended mask ( > 0x1FFFFFFF )

stdGlobalMask	Valid range from 0 to 0x7FF, used for message objects with 11 bit identifiers

extGlobalMask	Valid range from 0 to 0x1FFFFFFF, used for message objects with 29 bit identifiers

The mask registers allow message identifiers to be masked. A ‘0’ in a bit position is considered a don’t care and will allow an incoming message identifier with a ‘0’ or ‘1’ in the corresponding bit position to be accepted. A ‘1’ in a bit position only allows a ‘1’ in the incoming message identifier bit position. This allows a range of message identifiers to be received.

For example, if the stdGlobalMask is set to = 0x7FC and the message object #1 has an identifier set to 0x100, then message object #1 will receive incoming messages with identifiers 0x100, 0x101, 0x102, and 0x103.

## Function:	qcSetupMessage

The qcSetupMessage configures a message object.

Calling Conventions

C	unsigned int qcSetupMessage( 	unsigned char MessageNo,

unsigned char Direction,

long ID,

unsigned int Idtype,

unsigned char MessageLen )

Pascal		function qcSetupMessage( MessageNo: Byte;

Direction: Byte;

ID: longint;

IDtype: integer;

MessageLen: Byte ): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission or reception.

Direction:	1 = Transmit, 0 = Receive

ID:		Message identifier, standard identifier uses lowest 11 bits, extended uses lowest 29 bits.

IDtype:		0=Standard identifier( 11 bit id’s), 1=Extended identifier (29 bit id’s)

MessageLen:	Number of data bytes to send. Valid range 0 to 8.

Function Return Codes:	qcSetupMessage = 0	Successful

qcSetupMessage = -1	Port address not initialized

qcSetupMessage = -2	Invalid direction( > 1 )

qcSetupMessage = -9	Invalid message object number( < 1 or >15)

qcSetupMessage = -10	Invalid message length ( > 8 )

qcSetupMessage = -11	Invalid id type ( > 1 )

Example:

qcSetupMessage( 2, 1, 0x180, 0, 8 );	// set message object 2 for standard,

//id=0x180, transmit 8 bytes

## Function:	qcCheckMessage

The qcCheckMessage function reads the status of a message object. Use this function to poll for a received message.

Calling Conventions

C		unsigned int qcCheckMessage(unsigned char MessageNo )

Pascal		function qcCheckMessage( MessageNo: Byte ): integer; stdcall;

Parameters

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission or reception.

Function Return Codes:		qcCheckMessage = -1	Port address not initialized

qcCheckMessage = -9	Invalid message object number

qcCheckMessage >= 0	Use following bit definition

qcCheckMessage 		= 1xxxxxxx	Bit 7; Message Valid=1, invalid=0.

= x0xxxxxx	Bit 6; Transmit Interrupt, reference only, not used

= xx0xxxxx	Bit 5; Receive Interrupt, reference only, not used

= xxx0xxxx	Bit 4; Interrupt Pending, reference only, not used

= xxxx1xxx	Bit 3; Remote Frame Pending

= xxxxx1xx	Bit 2; Transmit Request

= xxxxxx1x	Bit 1; Message Lost

= xxxxxxx1	Bit 0; New data

Example:

x = qcCheckMessage( 1 );

if( x & 0x01 )

{	printf( “Message #1 received\n.”  );

y = qcDataReceivedLen( 1 );

for( z=0; z<y; z++ )

printf( “Data Byte %d = %d\n”, z, qcReadMessageData( 1, z ) );

qcResetMessage( 1 );		// allow another message to be received.

}

## Function:	qcReadMessage

The qcReadMessage reads the data from a message object into a array. The function qcReadMessage reads the number of data bytes received and places them at the address of the array passed to the function. Returns the number of bytes received. This function is useful for ‘C’ applications.

Calling Conventions

C		unsigned int qcReadMessage( 	unsigned char MessaageNo,

unsigned char *Dpointer )

Pascal		function qcReadMessage( MessageNo: Byte;

Dpointer: Pointer): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

Dpointer:	address pointer of array to received data into

Function Return Codes:	qcReadMessage >=0	Successful, returns number of bytes received

qcReadMessage = -1	Port address not initialized

qcReadMessage = -2	Invalid direction( > 1 )

qcReadMessage = -9	Invalid message object number( < 1 or >15)

## Function:	qcResetMessage

The qcResetMessage resets the message received (NewDat) flag for a message object.

Calling Conventions

C		unsigned int qcResetMessage(unsigned char MessageNo )

Pascal		function qcSendMessage( MessageNo: Byte ): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

## Function:	qcSendMessage

The qcSendMessage transmits a message.

Calling Conventions

C	unsigned int qcSendMessage( 	unsigned char MessageNo )

Pascal		function qcSendMessage( MessageNo: Byte ): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

Example:

qcSetupMessage( 1, 1, 0x180, 0, 8 );	// set message object 2 for standard,

//  id=0x180, transmit 8 bytes

for( x=0; x<8; x++ )

qcWriteMessageData( 1, x, 0 );	// clear the send data

qcSendMessage( 1 );			// send message in object #1

## Function:	qcOnline

The qcOnline function enables the CANPORT for network operations and must be executed before the network messages can be sent or received.

Calling Conventions

C		unsigned int qcOnline(void)

Pascal		function qcOnline: integer; stdcall;

Function Return Codes:		qcOnline = 0	Online successful

qcOnline = -1	Port address not initialized

## Function:	qcOffline

The qcOffline function disables the CANPORT from network operations.

Calling Conventions

C		unsigned int qcOffline(void)

Pascal		function qcOffline: integer; stdcall;

Function Return Codes:		qcOffline = 0	Offline successful

qcOffline = -1	Port address not initialized

## Function:	qcWriteMessageData

The qcWriteMessageData updates one data byte in a message object. Each message object contains up to 8 data bytes. This function allows access to the individual data bytes.

Calling Conventions

C		unsigned int qcWriteMessageData(unsigned char MessaageNo,

unsigned char DataNo,

unsigned char DataValue )

Pascal		function qcWriteMessageData( MessageNo: Byte;

DataNo: Byte;

DataValue: Byte): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

DataNo:	Data byte to write data to. Valid range 0 to 7 (8 bytes total)

DataValue:	Value to be written

Function Return Codes:		qcWriteMessageData = -1 Port address not initialized

qcWriteMessageData = -9 Invalid message object number

qcWriteMessageData = 0  Success

Example:	See qcSendMessage section.

## Function:	qcReadMessageData

The qcReadMessageData reads a data byte from the message object.

Calling Conventions

C		unsigned int qcReadMessageData(unsigned char MessaageNo,

unsigned char DataNo )

Pascal		function qcReadMessageData( MessageNo: Byte;

DataNo: Byte ): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

DataNo:	Data byte to read. Valid range 0 to 7 (8 bytes total)

Function Return Codes:		qcReadMessageData = -1 Port address not initialized

qcReadMessageData = -9 Invalid message object number

qcReadMessageData = x  Returns data byte value.

Example:	See qcCheckMessage section.

## Function:	qcDataReceivedLen

The qcDataReceivedLen returns the number of data bytes received.

Calling Conventions

C		unsigned int qcDataReceivedLen(unsigned char MessaageNo )

Pascal		function qcDataReceivedLen( MessageNo: Byte ): integer; stdcall;

Parameters:

MessageNo:	Valid range 1 to 15. 82527 message object to use for transmission.

Function Return Codes:		qcDataReceivedLen = -1 Port address not initialized

qcDataReceivedLen = -9 Invalid message object number

qcDataReceivedLen = x  Returns data byte value.

# CANPORT Sample Visual Basic Program

The sample application contains examples for using the CANPORT. It is written using Visual Basic version 5.0 for Windows 95 and does the following;

- Search for printer port address where CANPORT is attached

- Resets CANPORT

- Sets the baud rate to 500,000 bps

- Sets message objects

- Sends repetitive messages

- Interfaces common controls to message objects

This program shows how to send and receive messages between 2 PC’s or between 2 CANPORTS connected to different printer ports on 1 machine. One CANPORT is set to transmit the value of the scroll bar and a count using message ID 100 hex. The other CANPORT is set to receive the message and update it’s scroll bar and display the count value. A timer is used to send the message on a regular interval.

The files are located in the \canvb5 directory on the distribution diskette.

# CANPORT Delphi Application

The CANPORT Delphi application is contained under the \canport\candel directory. Delphi is object Pascal and is a visual design environment. The CANPORT.DLL was written using Delphi and is highly recommended as a development environment.

# Excel CANPORT Application

The included Excel spreadsheet shows how to write CAN applications using the Visual Basic for Applications that is part of Excel.

The spreadsheet contains the following sheets;

- Module1 - Macros written in Visual Basic

- Setup CANPORT - Sheet with initialization functions

- Messages - Sheet for sending and receiving messages

- Registers - Sheet for viewing the registers in the 82527

## Macros

The Macros sheet contains the Visual Basic code that is executed from Excel objects.

## Setup CANPORT

The Setup CANPORT sheet contains buttons to;

- Search for the CANPORT

- Reset CANPORT

- Set Baud Rate - parameters set on sheet

- Go online

Right click on the BUTTONS for the popup menu. The macro associated with the control is listed under the Assign Macro menu pick.

## Messages

The Messages sheet shows the details for sending messages.

To Send a Message:

- Enter the identifier in the A column

- Enter the message length in the B column

- Enter the required data bytes in columns C through J

- Select the entire row by clicking on the left row selector

- Push the Send Message button.

Results are displayed in the listbox.

The scroll bar gives an example of how to link the value of a control to a message. As the scroll bar is moved, the value is updated in the message in ROW 13, and this message is sent when the scroll bar is released (final value).

## Registers

The Registers sheet displays the internal registers in the 82527.

# DOS

## Libraries

Included on the distribution diskette under the \CANPORT\CANDOS directory are the following DOS libraries;

- CANDOSS.LIB - small library

- CANDOSM.LIB - medium library

- CANDOSL.LIB - large library

The libraries are built using Borland 4.5.

## Test Application

The source code for a test application to read all network traffic is contained under the CANDOS directory.